Purpose

The purpose of this assignment is to review system-level design through Verilog and the Basys-3 FPGA by creating a logical circuit that computes the factorial product of the input, displaying the given factorial number to the seven-segment FPGA display, and verifying its validity using waveform simulations.

Design Methodology

Using the six modules provided in the lab manual (register, counter, multiplexer, buffer, comparator, and multiplier), the Datapath (DP) and the Control Unit (CU) were designed. The CU has five inputs to the DP (EN, load_cnt, load_reg, sel0, and sel1), and two outputs from the DP (ERROR and DONE). The Algorithmic State Machine (ASM) was then constructed to determine which signals will be active and inactive during each state. The ASM chart is shown in Figure 1 below. The ASM provides a reference to coding the CU.v file and the DP design, shown in Figure 3, provides a reference to coding the DP.v file. Since the highest valid value in the design is 12, the output is required to have 32 bits. If any number larger than 12 is input, the ERROR flag will be set and the program will return to the idle state, state 0. Due to only having 4 digits present on the seven-segment display, the HILO_MUX.v provides the method of displaying the 8 digits by allowing the user to select either the high bits or the low bits to be displayed. The right-most switch on the FPGA board (V17) has been programmed to control the HILO_sel input. When the input is LOW, the 4-digit display will project the lower 16 bits of the value as 4 hexadecimal values, however, when the input is HIGH, the upper 16 bits will be displayed. The ERROR flag, DONE flag, as well as the factorial functionality works as expected with the exception of 0!. Switches W15, W17, W16, and V16 were used to provide a value input to the system.

Tasks Successfully Accomplished

- Design the system’s datapath using the six building blocks (counter, register, comparator, multiplier, multiplexer, and buffer).

- Draw the two-piece system block diagram which shows all the relevant signals for the Control Unit and the Datapath

- Create the ASM chart to describe the operations of the datapath

- Draw the bubble diagram to show the Next State Logic of the control unit

- Create an output table showing the output logic of the control unit

- Create the Verilog Design code for the factorial accelerator

- Create the testbench file and verified the code

Tasks Not Completed

- Zero Factorial

  - The expected results for an input of 0 was not accurately met with the experimental result. When using 0 as an input, the expected result is 1, however, the actual results our FPGA produced was a 0. This error is believed to lie within the Control Unit because as the input passes through to the multiply module, it multiplies 0 and 1 and produces the value 0, which is stored into the register and outputs it through the buffer. The State Machine states to keep the control signal sel0 as 1 to output the value in the register, however, sel0 should be set to 0 in order to output a value of 1. This is an edge case that was not discovered in time before the demonstration.

Conclusion

The factorial functionality was working as expected with the exception of 0!. This value did not consistently display the result to be 1, but instead displayed the result as a value of 0 approximately half of the time. The datapath works as intended, however, there were some complications regarding the control unit which may be the cause of the incorrect output for the input value of 0!. Overall, this lab utilizes the concepts of system-level design and provides a thorough opportunity for students to review the concepts as well as Verilog programming. Although there were complications within the experiment, the experiment was fully discussed and exercised to have a better understanding of system-level design methodologies.

Appendix

- Diagrams and Tables

Figure 1: Displaying the ASM Chart, describing the cycle-by-cycle operations of the datapath

Figure 2: Displaying the input and output signals of the Control Unit

Figure 3: Displaying the parts, signals, and paths of the Datapath section in the lab experiment

Figure 4: Displaying the bubble diagram for the next state logic of the control unit

Table 1: Displaying the output logic for the next state logic of the control unit

| State | Next State | GO | GT1 | GT12 | en | load_cnt | load_reg | sel0 | sel1 | done | error |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | x | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | x | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | x | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 2 | x | x | x | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 2 | 3 | x | 0 | x | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 2 | 2 | x | 1 | x | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 3 | 0 | x | x | x | 0 | 0 | 0 | 1 | 1 | 1 | 0 |

Figure 5: Displaying the lower four digits of 12!. Input bits B3, B2, B1, and B0 are assigned  to switches W15, W17, W16, and V16 respectively, which uses the bit value 1100. The output is an expected 0xFC00.

Figure 6: Displaying the upper four digits of 12!. The output is the expected value of 0x1C8C.

Figure 7: Displaying the ERROR and DONE flag being triggered when entering an input of 1101 (1310).

- Source Code

factorial_FPGA.v

module factorial_FPGA( input wire [3:0] IN, input wire GO, rst, clk100MHz, HILO_sel, output done, error, output wire [3:0] LEDSEL, output wire [6:0] LEDOUT ); wire clk_5KHz; wire dGO; wire [31:0] OUT; wire [3:0] dig0, dig1, dig2, dig3, dig4, dig5, dig6, dig7; wire [3:0] HEX3, HEX2, HEX1, HEX0; wire [6:0] LED3, LED2, LED1, LED0; clk_gen CLK ( .clk50MHz(clk100MHz), .rst(rst), .clk_5KHz(clk_5KHz)); button_debouncer BD(.clk(clk_5KHz), .button(GO), .debounced_button(dGO)); CU_DP the_controlunit(.IN(IN), .GO(GO), .RST(rst), .clk(clk_5KHz), .OUT(OUT), .done(done), .error(error)); bin2hex32 U1( .value(OUT), .dig0(dig0), .dig1(dig1), .dig2(dig2), .dig3(dig3), .dig4(dig4), .dig5(dig5), .dig6(dig6), .dig7(dig7) ); HILO_MUX DUT( .HI_dig3(dig7), .HI_dig2(dig6), .HI_dig1(dig5), .HI_dig0(dig4), .LO_dig3(dig3), .LO_dig2(dig2), .LO_dig1(dig1), .LO_dig0(dig0), .HILO_sel(HILO_sel), .HW_dig3(HEX3), .HW_dig2(HEX2), .HW_dig1(HEX1), .HW_dig0(HEX0) ); hex2led U_LD_3( .number(HEX3), .s0(LED3[0]), .s1(LED3[1]), .s2(LED3[2]), .s3(LED3[3]), .s4(LED3[4]), .s5(LED3[5]), .s6(LED3[6]) ); hex2led U_LD_2( .number(HEX2), .s0(LED2[0]), .s1(LED2[1]), .s2(LED2[2]), .s3(LED2[3]), .s4(LED2[4]), .s5(LED2[5]), .s6(LED2[6]) ); hex2led U_LD_1( .number(HEX1), .s0(LED1[0]), .s1(LED1[1]), .s2(LED1[2]), .s3(LED1[3]), .s4(LED1[4]), .s5(LED1[5]), .s6(LED1[6]) ); hex2led U_LD_0( .number(HEX0), .s0(LED0[0]), .s1(LED0[1]), .s2(LED0[2]), .s3(LED0[3]), .s4(LED0[4]), .s5(LED0[5]), .s6(LED0[6]) ); LED_MUX U3( .clk(clk_5KHz), .rst(rst), .LED0(LED0), .LED1(LED1), .LED2(LED2), .LED3(LED3), .LEDOUT(LEDOUT), .LEDSEL(LEDSEL)); endmodule

CU_DP.v

module CU_DP( input [31:0] IN, input GO, RST, clk, output [31:0] OUT, output done, error ); wire GT1, GT12, en, load_reg, load_cnt, sel0, sel1; CU control_unit(.GO(GO), .GT1(GT1), .GT12(GT12), .clk(clk), .rst(RST), .en(en), .load_reg(load_reg), .load_cnt(load_cnt), .sel0(sel0), .sel1(sel1), .done(done), .error(error)); DP datapath(.IN(IN), .clk(clk), .EN(en), .load_cnt(load_cnt), .load_reg(load_reg), .sel0(sel0), .sel1(sel1), .OUT(OUT), .GT1(GT1), .GT12(GT12)); endmodule

CU.v

module CU( input GO, GT1, GT12, clk, rst, output reg en, load_cnt, load_reg, sel0, sel1, done, error ); reg [1:0] state, next_state; initial begin next_state = 0; end always @(state, GO, GT1, GT12) begin case(state) 0: begin if (GO == 1) begin done = 0; error = 0; next_state = 1; end else next_state = 0; if (GT12 == 1) begin error = 1; done = 1; next_state = 0; end end 1: begin next_state = 2; end 2: begin if (GT1 == 0) begin next_state = 3; end else if (GT1 == 1) begin next_state = 2; end end 3: begin next_state = 0; done = 1; end endcase end always @(posedge clk, posedge rst) begin if (rst == 1) begin state <= 0; end else begin state <= next_state; end end always @(state) begin case(state) 0: begin en = 0; load_cnt = 0; load_reg = 0; sel0 = 0; sel1 = 1; end 1: begin en = 1; load_cnt = 1; load_reg = 1; sel0 = 0; sel1 = 1; end 2: begin en = 1; load_cnt = 0; load_reg = 1; sel0 = 1; sel1 = 1; end 3: begin en = 0; load_cnt = 0; load_reg = 0; sel0 = 1; sel1 = 1; end endcase end endmodule

DP.v

module DP( input wire [31:0] IN, input wire clk, EN, load_cnt, load_reg, sel0, sel1, output wire [31:0] OUT, output wire GT1, GT12 ); wire [31:0] Q_count, Q_reg, Q_mux, Q_mul; count #(32) counter(.D(IN), .load_cnt(load_cnt), .en(EN), .clk(clk), .Q(Q_count)); mux #(32) reg_mux(.A(1), .B(Q_mul), .sel(sel0), .OUT(Q_mux)); multiply #(32) multiplier(.X(Q_reg), .Y(Q_count), .Z(Q_mul)); register #(32) register0(.D(Q_mux), .clk(clk), .load_reg(load_reg), .Q(Q_reg)); compare #(32) compare_1(.A(Q_count), .B(1), .GT(GT1)); buffer #(32) buff(.A(0), .B(Q_reg), .sel(sel1), .OUT(OUT)); compare #(32) compare_12(.A(IN), .B(12), .GT(GT12)); endmodule

count.v

module count #(parameter WIDTH = 4)( input [WIDTH - 1:0] D, input load_cnt, en, clk, output reg [WIDTH - 1:0] Q ); always @ (posedge clk) if (en == 1) begin if (load_cnt == 0) begin Q <= Q - 1; end else begin Q <= D; end end else begin Q <= Q; end endmodule

mux.v

module mux #(parameter W = 4)( input [W-1:0] A, B, input sel, output reg [W-1:0] OUT ); always @ (A, B, sel) begin case (sel) 1'b0: OUT = A; 1'b1: OUT = B; endcase end endmodule

multiply.v

module multiply #(parameter WIDTH = 4)( input [WIDTH - 1:0] X, Y, output reg [WIDTH - 1:0] Z ); always @ (X or Y) begin Z = X * Y; end endmodule

register.v

module register #(parameter W = 4)( input [W-1:0] D, input clk, load_reg, output reg [W-1:0] Q ); always @(posedge clk) if (load_reg == 1) begin Q <= D; end endmodule

compare.v

module compare #(parameter WIDTH = 4)( input [WIDTH - 1:0] A, B, output reg GT ); always @ (A or B) begin if (A > B) begin GT = 1; end else begin GT = 0; end end endmodule

buffer.v

module buffer #(parameter W = 4)( input [W-1:0] A, B, input sel, output reg [W-1:0] OUT ); always @ (A, B, sel) begin case (sel) 1'b0: OUT = A; 1'b1: OUT = B; endcase end endmodule

constraints.xdc

## This file is a general .xdc for the Basys3 rev B board ## Clock signal set_property PACKAGE_PIN W5 [get_ports clk100MHz] set_property IOSTANDARD LVCMOS33 [get_ports clk100MHz] create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk100MHz] ## Switches set_property PACKAGE_PIN V17 [get_ports HILO_sel] set_property IOSTANDARD LVCMOS33 [get_ports HILO_sel] set_property PACKAGE_PIN V16 [get_ports {IN[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {IN[0]}] set_property PACKAGE_PIN W16 [get_ports {IN[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {IN[1]}] set_property PACKAGE_PIN W17 [get_ports {IN[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {IN[2]}] set_property PACKAGE_PIN W15 [get_ports {IN[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {IN[3]}] #set_property PACKAGE_PIN V15 [get_ports {IN[0]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[5]}] #set_property PACKAGE_PIN W14 [get_ports {sw[6]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[6]}] #set_property PACKAGE_PIN W13 [get_ports {sw[7]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[7]}] #set_property PACKAGE_PIN V2 [get_ports {sw[8]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[8]}] #set_property PACKAGE_PIN T3 [get_ports {sw[9]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[9]}] #set_property PACKAGE_PIN T2 [get_ports {sw[10]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[10]}] #set_property PACKAGE_PIN R3 [get_ports {sw[11]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[11]}] #set_property PACKAGE_PIN W2 [get_ports {sw[12]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[12]}] #set_property PACKAGE_PIN U1 [get_ports {sw[13]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[13]}] #set_property PACKAGE_PIN T1 [get_ports {sw[14]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[14]}] #set_property PACKAGE_PIN R2 [get_ports {sw[15]}] #set_property IOSTANDARD LVCMOS33 [get_ports {sw[15]}] ##7 segment display set_property PACKAGE_PIN W7 [get_ports {LEDOUT[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[0]}] set_property PACKAGE_PIN W6 [get_ports {LEDOUT[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[1]}] set_property PACKAGE_PIN U8 [get_ports {LEDOUT[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[2]}] set_property PACKAGE_PIN V8 [get_ports {LEDOUT[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[3]}] set_property PACKAGE_PIN U5 [get_ports {LEDOUT[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[4]}] set_property PACKAGE_PIN V5 [get_ports {LEDOUT[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[5]}] set_property PACKAGE_PIN U7 [get_ports {LEDOUT[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDOUT[6]}] #set_property PACKAGE_PIN V7 [get_ports dp] #set_property IOSTANDARD LVCMOS33 [get_ports dp] set_property PACKAGE_PIN U2 [get_ports {LEDSEL[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDSEL[0]}] set_property PACKAGE_PIN U4 [get_ports {LEDSEL[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDSEL[1]}] set_property PACKAGE_PIN V4 [get_ports {LEDSEL[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDSEL[2]}] set_property PACKAGE_PIN W4 [get_ports {LEDSEL[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {LEDSEL[3]}] # LEDs set_property PACKAGE_PIN U16 [get_ports {done}] set_property IOSTANDARD LVCMOS33 [get_ports {done}] set_property PACKAGE_PIN E19 [get_ports {error}] set_property IOSTANDARD LVCMOS33 [get_ports {error}] ##Buttons set_property PACKAGE_PIN U18 [get_ports rst] set_property IOSTANDARD LVCMOS33 [get_ports rst] set_property -dict {PACKAGE_PIN W19 IOSTANDARD LVCMOS33} [get_ports {GO}]; # Left Button

factorial_tb.v

module factorial_tb(); reg [31:0] tb_in; reg tb_clk, tb_go, tb_rst; wire [31:0] tb_out; wire tb_done, tb_error; reg [31:0] exp_out; reg exp_done; integer i, count; CU_DP DUT(.IN(tb_in), .GO(tb_go), .RST(tb_rst), .clk(tb_clk), .OUT(tb_out), .done(tb_done), .error(tb_error)); initial begin tb_rst = 0; i = 0; while (i < 16) begin exp_out = 1; tb_in = i; tb_go = 1; tick; exp_done = 0; while (tb_done == 0) begin tick; end exp_done = 1; count = i; while (count > 1) begin exp_out = exp_out * count; count = count - 1; end tick; i = i + 1; end $finish; end task tick; begin tb_clk = 0; #5; tb_clk = 1; #5; end endtask endmodule

bin2hex32.v

module bin2hex32( input wire [31:0] value, output wire [3:0] dig0, output wire [3:0] dig1, output wire [3:0] dig2, output wire [3:0] dig3, output wire [3:0] dig4, output wire [3:0] dig5, output wire [3:0] dig6, output wire [3:0] dig7 ); assign dig0 = value & 4'hFF; assign dig1 = value >> 4 & 4'hFF; assign dig2 = value >> 8 & 4'hFF; assign dig3 = value >> 12 & 4'hFF; assign dig4 = value >> 16 & 4'hFF; assign dig5 = value >> 20 & 4'hFF; assign dig6 = value >> 24 & 4'hFF; assign dig7 = value >> 28 & 4'hFF; endmodule

button_debouncer.v

module button_debouncer #(parameter depth = 16) ( input wire clk, /* 5 KHz clock */ input wire button, /* Input button from constraints */ output reg debounced_button ); localparam history_max = (2**depth)-1; /* History of sampled input button */ reg [depth-1:0] history; always @ (posedge clk) begin /* Move history back one sample and insert new sample */ history <= { button, history[depth-1:1] }; /* Assert debounced button if it has been in a consistent state throughout history */ debounced_button <= (history == history_max) ? 1'b1 : 1'b0; end endmodule

clk_gen.v

module clk_gen(clk50MHz, rst, clksec4, clk_5KHz); input clk50MHz, rst; output clksec4, clk_5KHz; reg clksec4, clk_5KHz; integer count, count1; always@(posedge clk50MHz) begin if(rst) begin count = 0; count1 = 0; clksec4 = 0; clk_5KHz =0; end else begin if(count == 100000000) begin clksec4 = ~clksec4; count = 0; end if(count1 == 20000) begin clk_5KHz = ~clk_5KHz; count1 = 0; end count = count + 1; count1 = count1 + 1; end end endmodule

HILO_MUX.v

module HILO_MUX( input wire [3:0] HI_dig3, input wire [3:0] HI_dig2, input wire [3:0] HI_dig1, input wire [3:0] HI_dig0, input wire [3:0] LO_dig3, input wire [3:0] LO_dig2, input wire [3:0] LO_dig1, input wire [3:0] LO_dig0, input wire HILO_sel, output wire [3:0] HW_dig3, output wire [3:0] HW_dig2, output wire [3:0] HW_dig1, output wire [3:0] HW_dig0 ); assign HW_dig3 = HILO_sel ? HI_dig3 : LO_dig3; assign HW_dig2 = HILO_sel ? HI_dig2 : LO_dig2; assign HW_dig1 = HILO_sel ? HI_dig1 : LO_dig1; assign HW_dig0 = HILO_sel ? HI_dig0 : LO_dig0; endmodule

LED_MUX.v

module LED_MUX (clk, rst, LED0, LED1, LED2, LED3, LEDOUT, LEDSEL); input clk, rst; input [6:0] LED0, LED1, LED2, LED3; output[3:0] LEDSEL; output[6:0] LEDOUT; reg [3:0] LEDSEL; reg [6:0] LEDOUT; reg [1:0] index; always @(posedge clk) begin if(rst) index = 0; else index = index + 1; end always @(index or LED0 or LED1 or LED2 or LED3) begin case(index) 0: begin LEDSEL = 4'b1110; LEDOUT = LED0; end 1: begin LEDSEL = 4'b1101; LEDOUT = LED1; end 2: begin LEDSEL = 4'b1011; LEDOUT = LED2; end 3: begin LEDSEL = 4'b0111; LEDOUT = LED3; end default: begin LEDSEL = 0; LEDOUT = 0; end endcase end endmodule

- Waveforms

The waveform simulations below demonstrate that the results are being outputted as expected. In Figure 8, the input to the system (tb_in) was a value of 4, which our expected value should be 4 x 3 x 2 x 1, which equals 24 (0x18). The waveform shows that at DONE, our experimental output (tb_out) is 0x18, which is the result we were expecting.

Figure 8: Displaying the Waveform Simulation signals from inputs 0 to 4. Here, we can see that the expected output for input 4 is 0x18 and our output (tb_out) is also 0x18. There are no errors and the DONE flag is raised

Figure 9: Displaying the Waveform Simulation from inputs 5 to 8.

Figure 10: Displaying the Waveform Simulation for inputs 9 to 11

Figure 11: Displaying the Waveform Simulation for inputs 12 to 15. Here, we can see that once 13, 14, and 15 are used as inputs, the ERROR flag (tb_error) is set to HIGH, making the output equal 0 due to never leaving State 0 of the State Machine.